ICP-MS Reprocessing, Exporting, and Printing Guide

Revised 9/26/2016

To reprocess in the CAL you will need to transfer the dataset and the method from the instrument computer to your program folder on Orca.  Copy from the ElanData folder:  C:/ElanData/Dataset/[your folder], Methods/[your method].  If you need any other files or files that were not used in the original run, copy those also.  The following instructions can be followed on the instrument computer or from the virtual Elan software in the CAL.

Opening the Elan Software through a Virtual Machine

- Click the Start button on the desktop.

- Click on All Apps, scroll down and click on ICPMS_ELAN_and_GCMSD_Chem.  This will open the virtual machine you will use to access the Elan Software.  It will take a moment to start up

  - If the system asks if the software has been moved or copied select Copied

- Click the Player button with the drop down arrow, select Manage

  - Select Virtual Machine Settings, Click the Options tab, highlight Shared Folders

  - On the right select the dot next to Always enabled

  - Make sure the Map as a network drive in Windows guests box is checked, click OK

  - In the "Folders" panel below, click Add & then Next

  - Browse for and highlight the folder containing your methods file & dataset. Click OK, Next

    - Ensure that the "Enable this share" box is checked, leave the "Read-only" box unchecked.

    - Click Finish, then OK to return to the Virtual Machine

  - Double check that the folder got mapped by checking under My computer in the virtual machine

    - The drive should be labeled as “Shared Folders on ‘vmware_host’”

- If not in full screen mode, click the “Enter Full Screen Mode” button

- Click the Start button on your virtual desktop, select Programs, ELAN DRC-e, click on the ELAN software icon

Reprocessing to Reproduce or Alter Calibration Curves and/or View (on monitor) or Print Single Data Files from a Dataset

- Open all of the appropriate files (Dataset, Report Template, and Methods) by clicking on the green R, click Load, and select the pathway (typically your program folder on Orca in Shared Folders on ‘vmware-host’).  *****Loading your dataset may take a couple of minutes****

- In most cases, before reprocessing, click on the Method button

  - Click on the Report tab, and make sure that the Send to Printer box is not checked

- Click on the Dataset button

  - To reprocess with all of the original conditions, click on the Use Original Conditions box.  Do not check this box if you changed or need to change something in your method or dataset

  - To modify Standard Order, ID or Read Type adjust the specific parameter within the dataset under the appropriate column

  - If not using original conditions, you may also choose to save the reprocessed results in a new file by checking the Save Reprocessed Data box. Save this to the program folder, not the virtual machine

  - Highlight the row numbers for the standards/blanks/sample(s) that you want reprocessed, and click Reprocess.  This must be done before any individual samples can be viewed or printed

    - To view the calibration curve(s) click the CalibView button

- To view a report for a single standard/blank/sample: (You must first reprocess your standard curve per step 8.d)

  - Highlight the row numbers for the standards/blanks/sample(s) that you want printed, and click Reprocess

- To print out individual data files: (Not recommended: see export options)

  - Click on the Method button, select the Report tab and check the Send to Printer box

  - Click on the Dataset button

    - Uncheck the Use Original Conditions box

    - Highlight the row numbers for the standards/blanks/sample(s) that you want printed, and click Reprocess

  - Do not save the method with the printer box checked

  - You will be presented with a save file prompt for an .xps file for each sample. Open the drop-down menu labeled "Save in:" and select "Shared Folders on 'vmware-host'" & navigate to your program folder. Name the file & click OK. Do this for each sample.

  - You can then print the .xps file(s) you want to print from the host (not the Virtual) machine.

Reprocessing to Export Complete Results for Each Sample into Excel (Best for data evaluation)

- If you have not already done this, open all of the appropriate files (i.e. Dataset, Report Template, and Methods) by clicking on the green R, click Load, and select the pathway (typically your program folder on Orca in the Shared Folders on ‘vmware-host’).  *****Loading your dataset may take a couple of minutes****

- Click on the Method button and then the Report tab

  - Make sure that the Send to Printer box is not checked

  - In the Report to File box

    - Check the box next to Send to File

    - Under the Report Option Template click on Browse select a template (Defaults:  Quant Summary.rop or TotalQuant Summary.rop).

    - Under Report Filename click on Browse

      - Open the drop-down menu next to Look in: to select a file pathway

        - This should be located in your program folder, not in the virtual machine

      - Enter a file name for the exported data, click Open

    - Check  Include Titles, Use Delimiter, Use Separator, and Append

- Click on the Dataset button, do not click Use Original Conditions box.

  - To create a report, highlight the row numbers for the standards/blanks/and samples to be included and click Reprocess

  - Do not save the method

Reprocessing to Export a Summary Report into Excel (NOTE:  This will not work with Total Quant data)

- If you have not already done this, open all of the appropriate files (i.e. Dataset, Report Template, and Methods) by clicking on the green R, click Load, and select the pathway (typically your program folder on Orca in the Shared Folders on ‘vmware-host’).  *****Loading your dataset may take a couple of minutes****

- Click on the Method button and then the Report tab

  - Make sure that the Send to Printer box is not checked

- Click on the Dataset button, do not click Use Original Conditions box.

  - Click on the Summary Report button

    - To display all results, select the summary by Analyte or Sample box (Summary by Analyte is very useful for plotting variations in internal standard response)

    - To display final concentrations only, select summary by Sample, and check the Cross Tab box (This provides a very compact file)

    - Under Output To, check the File box, uncheck the Print box

    - Click Browse to select the filename and file pathway for the exported data.

      - This should be located in your program folder, not in the virtual machine.

    - Press the OK button

  - To create the summary report, highlight the row numbers for the standards, blanks, and samples to be included, click the Reprocess button

  - Do not save the method

Opening Complete or Summary Report Files in Excel (Outside of the Virtual Machine)

- Open Excel, click on File, Open, select your folder or drive

  - To the right of the file name box, select All Files from the drop-down menu, select the file to be imported and then click Open

- A text import wizard will appear. Select the dot next to Delimited, click Next

  - Under Delimiters, select both Tab and Comma, click Next

  - Under Column Data Format select General, click Finish

- In excel, adjust final formatting as desired