Common Available Internal Standards

Element 						m/z

Sc – Scandium (monoisotopic)	45

Ga – Gallium					69, 71

Ge – Germanium				70, 72, 73, 74, 76

Y – Yttrium (monoisotopic)		89

In – Indium					113, 115

Tb – Terbium					159

Bi – Bismuth (monoisotopic)		209

Th – Thorium (monoisotopic)		232

Do not use Sc, Y or Rare Earth Elements (Lanthanide Series) as internal standards with samples/standards that have been digested in HF-.  They will form insoluble precipitates.